nuTENSA Treatment Protocols

How it Works:

The device uses RF (radio frequency) to improve skin firmness and elasticity. On a deeper level it can be used to help alleviate pain due to sports injury.

Immediate Results:

- Dermis: Direct contraction leads to tightening.

- Fibrous Septum: Contraction facilitates reshaping.

Long-Term Results (Secondary Wound Healing Response):

- Collagen deposition, remodeling, and shaping occur gradually over time.

Treatment Indications:

Skin Tightening

Collagen Stimulation

Wrinkle reduction

Fat loss

Weight loss

Pain rehabilitation

Sports Injury

Contraindications:

- Patients with acute inflammation, deep vein thrombosis, cancer, or similar medical conditions

- Metal implants in treatment area

- Epilepsy or seizure history

- Skin conditions in treatments area such as: Ezema, Open Wounds, Active Infection

- Treatment on Abdomen with Presence of Hernia

- Individuals with numbness or inability to sense heat

- Pregnancy or lactating

Client Preparation

- Avoid heavy meals 1 hour prior

- Hydrate well before and after

- Remove metal jewelry and/or electronic devices such as: Apple Watch/ Garmin

- Take pre-treatment photos or run body analysis on nuSCALE

*RF Cream is the conductor to be used for treatment*

IMPORTANT:  YOU MUST USE A GROUNDING PLATE FOR CET AND RET.

YOU MUST STOP TREATMENT WHEN ANSWERING THE PHONE.

#### Treatment Parameters (Standard Protocols)

| Area | Duration | Intensity | Frequency | Number of Sessions |
| --- | --- | --- | --- | --- |
| Abs | 30-45 min | Moderate to High (50-60%) | 1x/week | 8 sessions |
| Glutes/ Hamstrings | 30-45 min | Moderate to High (50-60%) | 1x/week | 8 sessions |
| Quads/ Inner Thigh | 30-45 min | Moderate (45-55 %) | 1x/week | 8 sessions |
| Arms | 30 min | Low to Moderate (30-50%) | 1x/week | 8 sessions |
| Face/Neck | 20-30 min | Low to Moderate (30-50 %) | 1x/week | 8 sessions |

The duration of the session will start counting after you have reached the correct temperature for that particular client. This may vary and can take 2- 5 mins.

Adjust intensity based on client tolerance. Always start lower and gradually increase.

You may also choose to treat 8 sessions in the format of 1x/week for the first 4 weeks, then 2x/week for the next 2 weeks.

Post- Treatment Care

- Encourage hydration

- Mild soreness is normal

- Space sessions 7-10 days apart to allow for body to recover and begin the natural process of excretion

- Every 3rd session redo body analysis on nuSCALE

Recommendation: Maintenance treatments should be every 2-3 months per indication.